Meter Data Collection

Requirements Document

V5

## Contents

Contents	1

Meter-Client communication Association with Low Level security Use-Case (LLS):	3

Instantaneous data read:	5

Time synchronization:	6

Wake Up Sims:	6

Monthly Billing Data:	8

Optical Port Open/Close:	10

Billing Data (without Block Transfer):	11

Billing Data(block transfer):	12

Instantaneous Data Profile without Block Transfer:	13

Instantaneous Profile with Block transfer:	14

Load Profile without Block Transfer (Single Entry):	15

Load Profile Block Transfer:	15

On Demand parameter read:	17

1-	DMDT (Device Meta Data):	17

2-	WSIM (Wakeup Sim ID)	17

3-	IPPO (Programmed IP and Port)	17

4-	MDSM (Programmed Meter Data Sampling)	17

5-	OPPO (Programmed Optical Port)	18

6-	MSIM (SIM -ID of Sim inside meter)	18

7-	AUXR (Aux Relay status)	18

8-	Programmed Sanctioned Load:	18

9-	Programmed Load shedding schedule:	18

10-	Programmed Time of Use:	19

Device Creation:	19

Update Meta Data:	20

Update MDI Reset Date:	20

Update IP Port:	20

Meter Data Sampling:	20

Aux Relay operation:	21

Sanctioned Load Control:	21

Load_shedding_Schedule:	22

Paramterization_Cancellation:	22

SANC:	22

LSCH:	22

TIOU:	22

C7 01 C1 00 00	23

Appendix I – DLMS Abbreviations	23

Figures	25

AARQ request format:	25

Application Layer Services summary for LN referencing:	25

State Machine for the client Side Control Function:	26

State Machine for Server side Control Function:	26

# Meter-Client communication Association with Low Level security Use-Case (LLS):

Heart Beat from Meter: DD 04 81 BA A7 B8

Heartbeat from MDC : DA

Programmer AARQ:

00 01 00 30 00 01 00 38 60 36 A1 09 06 07 60 85 74 05 08 01 01 8A 02 07 80 8B 07 60 85 74 05 08 02 01 AC 0A 80 08 31 32 33 34 35 36 37 38 BE 10 04 0E 01 00 00 00 06 5F 1F 04 00 00 7E 1F 04 B0

Response AARE

00 01 00 01 00 30 00 2B 61 29 A1 09 06 07 60 85 74 05 08 01 01 A2 03 02 01 00 A3 05 A1 03 02 01 00 BE 10 04 0E 08 00 06 5F 1F 04 00 00 1C 1F 01 2C 00 07

RLRQ Sent:  00 01 00 30 00 01 00 05 62 03 80 01 00

RLRE:  Received:  00 01 00 01 00 30 00 05 63 03 80 01 00

Error in Release: 00 01 00 01 00 30 00 03 D8 01 01

Auto Event Received:

Reply : 00 01 00 01 00 01 00 01 DB

# Instantaneous data read:

UDIL 3.1 code : 3.1.1

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 07 01 00 5E 5C 0A FF 02 00

Receive: 00 01 00 01 00 30 00 53 C4 01 81 00 01 01 02 11 09 0C 07 D0 01 01 06 01 34 36 00 AA 01 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 00 00 12 55 BA 12 00 00 10 03 E8 10 03 E8 10 03 E8 10 03 E8 12 13 75 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 11 02

Signal Strength:

Send:  00 01 00 30 00 01 00 0D C0 01 81 00 01 00 00 60 0C 05 FF 02 00

Receive:  00 01 00 01 00 30 00 07 C4 01 81 00 09 02 03 03 A7

Total Active Energy:

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 03 01 00 80 07 00 FF 02 00

Receive: 00 01 00 01 00 30 00 09 C4 01 81 00 06 00 00 00 00

Total Reactive Energy:

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 03 01 00 0F 07 00 FF 02 00

Receive: 00 01 00 01 00 30 00 09 C4 01 81 00 06 00 00 00 00

# Time synchronization:

UDIL 3.1 code : 3.3.2

Read:

Send : 00 01 00 30 00 01 00 0D C0 01 81 00 01 01 00 00 09 02 FF 02 00

Rcv: 00 01 00 01 00 30 00 09 C4 01 81 00 09 05 07 E6 07 19 01

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 01 01 00 00 07 01 FF 02 00

RCV: 00 01 00 01 00 30 00 0A C4 01 81 00 09 04 0F 04 2A FF

Write:

Send: 00 01 00 30 00 01 00 1B C1 01 81 00 08 00 00 01 00 00 FF 02 00 09 0C 07 E6 07 19 01 0E 3B 0E 00 80 00 00

RCV: c

# Wake Up Sims:

UDIL 3.1 code : 3.3.10

Read :

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 01 00 00 60 0C 80 FF 02 00

RCV: 00 01 00 01 00 30 00 11 C4 01 81 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 01 00 00 60 0C 81 FF 02 00

Rcv: 00 01 00 01 00 30 00 11 C4 01 81 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 01 00 00 60 0C 82 FF 02 00

RCV: 00 01 00 01 00 30 00 11 C4 01 81 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

Write:

Send : 00 01 00 30 00 01 00  1A C1 01 81 00 01 00 00 60 0C 80 FF 02 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

RCV: 00 01 00 01 00 30 00 04 C5 01 81 00

Send: 00 01 00 30 00 01 00 1A C1 01 81 00 01 00 00 60 0C 81 FF 02 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

RCV: 00 01 00 01 00 30 00 04 C5 01 81 00

Send: 00 01 00 30 00 01 00 1A C1 01 81 00 01 00 00 60 0C 82 FF 02 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

RCV : 00 01 00 01 00 30 00 04 C5 01 81 00

# Block transfer Frame Format Example:

C4 02 81

00

00 00 00 02

00 82

01 18

01 02 02 26 09 0C 07 E6 09 15 03 0A 00 00 00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 00 00 06 00 00 00 00 06

00 00 00 00 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06

00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E 02 26 09 0C 07 E6 09 15 03 0A 0A 00 00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 00 00 06 00 00 00 00 06

| Byte length tag | Byte Length | Position | No Bytes |
| --- | --- | --- | --- |
| 00 | 72 | 09 | 1 |
| 00 81 | 72 | 09, 10 | 1 |
| 00 82 | 01 18 | 09, 10 | 2 |

If Position(10)== 81

{

Position (12) = data

}

Else IF Position(10)==82

{

Position (13) = data

}

Else if position (9)==00 && position (10)!= 81

{

Position (12) = data

}

# Monthly Billing Data:

UDIL 3.2

Following is Monthly billing data details Current Billing data is the latest reset billing data while Billing data 1 and Billing data 2 is respectively old.

Current Billing Data:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 08 12 04

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 0F 14 23 FF

C0 01 81 00 01 00 00 60 0E 01 FF 02 00

C4 01 81 00 11 03

C0 01 81 00 07 01 00 62 01 00 FF 02 01 02 02 04 06 00 00 00 01 06 00 00 00 01 12 00 01 12 00 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 01 02 53 09 0C 07 E6 08 12 04 0F 12 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 0F ED 06 00 00 45 D5 06 00 00 00 00 06 00 00 00 00 06 00 00 55 C2 06 00 00 0F ED 06 00 00 45 D5 06 00 00 00 00 06 00 00 00 00 06 00 00 55 C2 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 E0 06 00 00 03 E1 06 00 00 00 00 06 00 00 00 00 06 00 00 04 C1 06  00 00 00 E0 06 00 00 03 E1 06 00 00 00 00 06 00 00 00 00 06 00 00 04 C1 06 00 00 00 E0 06 00 00 03 E1 06 00 00 00 00 06 00 00 00 00 06 00 00 04 C1 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00

C0 02 81 00 00 00 01

C4 02 81 00 00 00 00 02 00 82 01 18 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 2B 09 0C 07 E6 08 12 04 0F 12 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 2B 09 0C 07 E6 08 12 04 0F 12 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 2B 09 0C 07 E6 08 12 04 0F 12 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 2B 09 0C 07 E6 08 12 04 0F 12 00 00 80 00 FF 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 04 56 06 00 00 05 26 06 00 00 00 00 06 00 00 00 00 06 00 00 05 26 06 00 00 04 56 06 00 00 05 26 06 00 00 00 00 06 00 00 00 00 06 00 00 05 26 06 00 00 00 03 09 0C

C0 02 81 00 00 00 02

C4 02 81 01 00 00 00 03 00 0C 07 E6 08 12 04 0F 12 00 00 80 00 FF

Billing Data 2:

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00 8C 6D 7E

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 08 13 05

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 0D 0A 02 FF

C0 01 81 00 01 00 00 60 0E 01 FF 02 00

C4 01 81 00 11 03

C0 01 81 00 07 01 00 62 01 00 FF 02 01 02 02 04 06 00 00 00 02 06 00 00 00 02 12 00 01 12 00 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 01 02 53 09 0C 07 E6 08 12 04 0F 0C 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 0F ED 06 00 00 45 C8 06 00 00 00 00 06 00 00 00 00 06 00 00 55 B5 06 00 00 0F ED 06 00 00 45 C8 06 00 00 00 00 06 00 00 00 00 06 00 00 55 B5 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 E0 06 00 00 03 E0 06 00 00 00 00 06 00 00 00 00 06 00 00 04 C0 06 00 00 00 E0 06 00 00 03 E0 06 00 00 00 00 06 00 00 00 00 06 00 00 04 C0 06 00 00 00 E0 06 00 00 03 E0 06 00 00 00 00 06 00 00 00 00 06 00 00 04 C0 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00

C0 02 81 00 00 00 01

C4 02 81 00 00 00 00 02 00 82 01 18 00 00 00 00 00 80 00 FF 06 00 00 04 56 09 0C 07 E6 08 11 03 14 00 00 00 80 00 FF 06 00 00 04 FB 09 0C 07 E6 08 12 04 0C 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 04 FB 09 0C 07 E6 08 12 04 0C 00 00 00 80 00 FF 06 00 00 04 56 09 0C 07 E6 08 11 03 14 00 00 00 80 00 FF 06 00 00 04 FB 09 0C 07 E6 08 12 04 0C 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 04 FB 09 0C 07 E6 08 12 04 0C 00 00 00 80 00 FF 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 04 56 06 00 00 04 FB 06 00 00 00 00 06 00 00 00 00 06 00 00 04 FB 06 00 00 04 56 06 00 00 04 FB 06 00 00 00 00 06 00 00 00 00 06 00 00 04 FB 06 00 00 00 02 09 0C

C0 02 81 00 00 00 02

C4 02 81 01 00 00 00 03 00 0C 07 E6 08 12 04 0F 0C 00 00 80 00 FF

Billing Data 3:

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 08 13 05

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 10 26 27 FF

C0 01 81 00 01 00 00 60 0E 01 FF 02 00

C4 01 81 00 11 03

C0 01 81 00 07 01 00 62 01 00 FF 02 01 02 02 04 06 00 00 00 03 06 00 00 00 03 12 00 01 12 00 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 01 02 53 09 0C 07 E6 08 10 02 11 32 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00

C0 02 81 00 00 00 01 73 7F 7E

C4 02 81 00 00 00 00 02 00 82 01 18 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00

00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 07 D0 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 09 0C

C0 02 81 00 00 00 02 E8 4D 7E

C4 02 81 01 00 00 00 03 00 0C 07 E6 08 10 02 11 32 00 00 80 00 FF

# Monthly Billing Data (Block Transfer) Read between start date and end date time:

[ 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF] is the string of length 0C. For next block read programmer need to keep this and 06 data tag in mind.

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 08 12 04

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 0F 14 23 FF

C0 01 81 00 01 00 00 60 0E 01 FF 02 00

C4 01 81 00 11 03

C0 01 C1 00 07 01 00 62 01 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E7 03 03 FF 0F 28 00 FF FE D4 00 09 0C 07 E7 03 03 FF 0F 2C 00 FF FE D4 00 01 00

C4 02 C1 00 00 00 00 01 00 82 01 18

01 01 02 53 09 0C 07 E7 04 04 02 0F 0E 00 00 FE D4 00

06 00 00 00 01 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00

09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00

09 0C 00 00 00 00

C0 02 81 00 00 00 01

C4 02 C1 00 00 00 00 02 00 82 01 18

00 00 00 00 00 80 00 FF

06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF 06 00 00 00 00 09 0C 00 00 00 00 00 00 00 00 00 80 00 FF

06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 03 09 0C

C0 02 81 00 00 00 02

C4

02 C1 01

00 00 00 03

00

0C

07 E7 04 04 02 0E 00 00 00 80 00 FF

# Optical Port Open/Close:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

Read:

C0 01 81 00 01 00 00 60 3C 10 FF 02 00

C4 01 81 00 02 02 09 0C 07 D0 01 01 06 00 00 00 00 80 00 FF 09 0C 07 E9 01 01 03 00 00 00 00 80 00 FF

Write:

C1 01 81 00 01 00 00 60 3C 10 FF 02 00 02 02 09 0C 07 D0 01 01 06 00 00 00 00 80 00 FF 09 0C 07 E9 01 01 03 00 00 00 00 80 00 FF

C5 01 81 00

# Billing Data (without Block Transfer):

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 09 15 03

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 0B 0F 3B FF

C0 01 81 00 07 01 00 63 02 00 FF 03 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 26 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 02 04 12 00 07 09 06 01 00 63 02 00 FF 0F 04 12 00 00 02 04 12 00 03 09 06 01 00 01 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01

C0 02 81 00 00 00 01

C4 02 81 00 00 00 00 02 00 82 01 18 00 04 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 02 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 02 02 FF 0F 02 12 00 00

C0 02 81 00 00 00 02

C4 02 81 01 00 00 00 03 00 7E 02 04 12 00 04 09 06 01 00 0F 02 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 02 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 02 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 02 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 02 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 02 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 02 00 FF 0F 02 12 00 00

C0 01 81 00 07 01 00 63 02 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E6 09 15 03 0A 00 00 00 80 00 00 09 0C 07 E6 09 15 03 0A 05 00 00 80 00 00 01 00

C4 01 81 00 01 01 02 29

09 0C 07 E6 09 15 03 0A 00 00 00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06 00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00

# Billing Data (block transfer):

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 09 15 03

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 0B 32 16 FF

C0 01 81 00 07 01 00 63 02 00 FF 03 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 26 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 02 04 12 00 07 09 06 01 00 63 02 00 FF 0F 04 12 00 00 02 04 12 00 03 09 06 01 00 01 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01

C0 02 81 00 00 00 01

C4 02 81 00 00 00 00 02 00 82 01 18 00 04 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 02 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 02 02 FF 0F 02 12 00 00

C0 02 81 00 00 00 02

C4 02 81 01 00 00 00 03 00 7E 02 04 12 00 04 09 06 01 00 0F 02 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 02 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 02 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 02 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 02 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 02 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 02 00 FF 0F 02 12 00 00

C0 01 81 00 07 01 00 63 02 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E6 09 15 03 0A 00 00 00 80 00 00 09 0C 07 E6 09 15 03 0A 0A 00 00 80 00 00 01 00

C4 02 81

00 00 00 00 01

00 82

01 18

01 02 02 26 09 0C 07 E6 09 15 03 0A 00 00 00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 00 00 06 00 00 00 00 06

00 00 00 00 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06

00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E

06 00 00 00 00

06 00 00 00 00

06 00 00 00 00

02 26 09 0C 07 E6 09 15 03 0A 0A 00 00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 5C 06 00 00 7F 09 06 00 00 1B AD 06 00 00 63 5C 06

C0 02 81 00 00 00 01

C4 02 81

01

00 00 00 02

00 81

9A

00 00 7F 09 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06 00 00 00 00 06 00 00 00 01 06 00 00 00 01 06 00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 07 F0 06 00 00 09 7E 06 00 00 09 7E 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00

# Instantaneous Data Profile without Block Transfer:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 09 13 01

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 0C 38 21 FF

C0 01 81 00 07 01 00 63 01 01 FF 03 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 11 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 02 04 12 00 07 09 06 01 00 63 01 01 FF 0F 04 12 00 00 02 04 12 00 03 09 06 01 00 01 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0D 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 20 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 34 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 48 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 1F 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 33 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 47 07 00 FF 0F 02 12 00 00 02 04 12 00 01 09 06 00

C0 02 81 00 00 00 01

C4 02 81 01 00 00 00 02 00 1C 00 60 0C 05 FF 0F 02 12 00 00 02 04 12 00 01 09 06 00 00 60 0E 00 FF 0F 02 12 00 00

C0 01 81 00 07 01 00 63 01 01 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E6 09 13  03 01 0C 1E 00 80 00 00 09 0C 07 E6 09 13  03 01 0C 23 00 80 00 00 01 00

C4 01 81 00 01 03 02 11

09 0C

07 E7 06 1B 02 0E 00 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 6A 06 00 00 03 E8 12 00 00 12 00 00 12 58 07 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 A7 11 02

02 11

09 0C 07 E7 06 1B 02 0E 01 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 71 06 00 00 03 E8 12 00 00 12 00 00 12 58 3F 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 AF 11 02 02 11

09 0C 07 E7 06 1B 02 0E 02 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 6A 06 00 00 03 E8 12 00 00 12 00 00 12 58 05 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 B1 11 02

# Instantaneous Profile with Block transfer:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 09 13 01

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 0D 21 09 FF

C0 01 81 00 07 01 00 63 01 01 FF 03 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 11 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 02 04 12 00 07 09 06 01 00 63 01 01 FF 0F 04 12 00 00 02 04 12 00 03 09 06 01 00 01 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0D 1B 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 20 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 34 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 48 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 1F 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 33 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 47 07 00 FF 0F 02 12 00 00 02 04 12 00 01 09 06 00

C0 02 81 00 00 00 01

C4 02 81 01 00 00 00 02 00 1C 00 60 0C 05 FF 0F 02 12 00 00 02 04 12 00 01 09 06 00 00 60 0E 00 FF 0F 02 12 00 00

C0 01 81 00 07 01 00 63 01 01 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E6 09 13 03 01 0C 1E 00 80 00 00 09 0C 07 E6 09 13 03 01 0C 28 00 80 00 00 01 00

C4 02 81

00

00 00 00 01

00 82

01 18

01 08

02 11 09 0C 07 E7 06 1B 02 0E 00 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 6A 06 00 00 03 E8 12 00 00 12 00 00 12 58 07 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 A7 11 02

02 11 09 0C 07 E7 06 1B 02 0E 01 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 71 06 00 00 03 E8 12 00 00 12 00 00 12 58 3F 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 AF 11 02

02 11 09 0C 07 E7 06 1B 02 0E 02 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 6A 06 00 00 03 E8 12 00 00 12 00 00 12 58 05 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 B1 11 02

02 11 09 0C 07 E7 06 1B 02 0E 03 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00

C0 02 81 00 00 00 01

C4 02 81 00

00 00 00 02

00 82

01 18

06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 7C 06 00 00 03 E8 12 00 00 12 00 00 12 58 52 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 A9 11 02

02 11 09 0C 07 E7 07 15 05 11 36 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 77 06 00 00 03 E8 12 00 00 12 00 00 12 58 47 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 A7 11 02

02 11 09 0C 07 E7 07 15 05 11 37 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 75 06 00 00 03 E8 12 00 00 12 00 00 12 58 59 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 A7 11 02

02 11 09 0C 07 E7 07 15 05 11 38 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 6A 06 00 00 03 E8

C0 02 81 00 00 00 02

C4 02 81 01

00 00 00 03

00

72

12 00 00 12 00 00 12 58 07 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 A7 11 02

02 11 09 0C 07 E7 07 15 05 11 39 00 00 FF 10 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 6A 06 00 00 03 E8 12 00 00 12 00 00 12 58 07 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 09 02 03 A7 11 02

# Load Profile without Block Transfer (Single Entry):

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 09 17 05

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 10 2F 2E FF

C0 01 81 00 07 01 00 63 01 00 FF 03 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 25 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 02 04 12 00 07 09 06 01 00 63 01 00 FF 0F 04 12 00 00 02 04 12 00 03 09 06 01 00 01 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01

C0 02 81 00 00 00 01

C4 02 81 00 00 00 00 02 00 82 01 18 00 04 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 07 00 FF 0F 02 12 00 00

C0 02 81 00 00 00 02

C4 02 81 01 00 00 00 03 00 6C 02 04 12 00 03 09 06 01 00 0F 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0E 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0D 1B 00 FF 0F 02 12 00 00

C0 01 81 00 07 01 00 63 01 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E6 09 17 05 10 00 00 00 80 00 00 09 0C 07 E6 09 17 05 10 05 00 00 80 00 00 01 00

C4 01 81 00 01 01 02 25 09 0C 07 E6 09 17 05 10 00 00 00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 6E 06 00 00 7F 1B 06 00 00 1B AD 06 00 00 63 6E 06 00 00 7F 1B 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 0A 06 00 00 00 0A 06 00 00 00 00 06 00 00 00 0A 06 00 00 00 0A 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 A9 06 00 00 03 E8

# Load Profile Block Transfer:

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 09 17 05

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 11 0C 0C FF

C0 01 81 00 07 01 00 63 01 00 FF 03 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 25 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 02 04 12 00 07 09 06 01 00 63 01 00 FF 0F 04 12 00 00 02 04 12 00 03 09 06 01 00 01 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0F 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01

C0 02 81 00 00 00 01

C4 02 81 00 00 00 00 02 00 82 01 18 00 04 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 08 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 01 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 02 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 08 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 01 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 02 06 00 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 01 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 02 FF 0F 02 12 00 00 02 04 12 00 04 09 06 01 00 0F 06 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 01 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 02 07 00 FF 0F 02 12 00 00

C0 02 81 00 00 00 02

7E A0 85 61 98 90 00 23 30 67 B9 E6 E7 00 C4 02 81 01 00 00 00 03 00 6C 02 04 12 00 03 09 06 01 00 0F 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 03 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 04 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 80 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0E 07 00 FF 0F 02 12 00 00 02 04 12 00 03 09 06 01 00 0D 1B 00 FF 0F 02 12 00 00

C0 01 81 00 07 01 00 63 01 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E6 09 17 05 10 00 00 00 80 00 00 09 0C 07 E6 09 17 05 10 0F 00 00 80 00 00 01 00

C0 01 81 00 07 01 00 63 01 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E7 09 0C 05 13 32 02 00 80 00 00 09 0C 07 E7 09 0D 05 13 32 02 00 80 00 00 01 00

C4 02 81

00

00 00 00 01

00 82

01 18

01 02

02 25 09 0C 07 E6 09 17 05 10 00 00

00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 6E 06 00 00 7F 1B 06 00 00 1B AD 06 00 00 63 6E 06 00 00 7F 1B 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00

01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 0A 06 00 00 00 0A 06 00 00 00 00 06 00 00 00 0A 06 00 00 00 0A 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 A9 06 00 00 03 E8

02 25 09 0C 07 E6 09 17 05 10 0A 00

00 FF 10 00 06 00 00 02 58 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 1B AD 06 00 00 63 6E 06 00 00 7F 1B 06 00 00 1B AD 06 00 00 63 6E 06 00 00 7F 1B 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00

C0 02 81 00 00 00 01

C4 02 81

01

00 00 00 02

00

6E

01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 01 1E 06 00 00 04 CD 06 00 00 05 EB 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 0A 06 00 00 00 0A 06 00 00 00 00 06 00 00 00 0A 06 00 00 00 0A 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 06 00 00 00 00 12 13 A9 06 00 00 03 E8

# Event Read Without Block Transfer:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

All Event codes are according to UDIL document description.

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 0B 08 02

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 06 33 0A FF

C0 01 81 00 01 00 00 60 0E 01 FF 02 00

C4 01 81 00 11 04

C0 01 81 00 07 01 00 63 62 00 FF 02 00

C4 01 81 00 01 0C 02 02 10 00 DA 09 0C 07 E6 0B 08 FF 04 22 38 00 80 00 FF 02 02 10 00 CF 09 0C 07 E6 0B 08 FF 04 23 25 00 80 00 FF 02 02 10 00 85 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 87 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 71 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 05 0D

2A 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 05 0D 2A 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 06 33 08 00 80 00 FF

# Event Read With Block Transfer:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

Event Frame is [ 02 02 10 00 DA 09 0C 07 E6 0B 08 FF 04 22 38 00 80 FF]

C0 01 81 00 01 00 00 60 3C 07 FF 02 00

C4 01 81 00 09 01 FF

C0 01 81 00 01 00 00 60 01 00 FF 02 00

C4 01 81 00 09 0C 30 30 33 30 39 38 30 30 30 30 30 31

C0 01 81 00 01 00 00 60 01 04 FF 02 00

C4 01 81 00 09 07 54 50 49 2D 33 34 47

C0 01 81 00 01 01 00 00 09 02 FF 02 00

C4 01 81 00 09 05 07 E6 0B 0F 02

C0 01 81 00 01 01 00 00 09 01 FF 02 00

C4 01 81 00 09 04 10 1B 01 FF

C0 01 81 00 01 00 00 60 0E 01 FF 02 00

C4 01 81 00 11 04

C0 01 81 00 07 01 00 63 62 00 FF 02 00

C4 02 81

00

00 00 00 01

00 82

01 18

01 14 02 02 10 00 DA 09 0C 07 E6 0B 08 FF 04 22 38 00 80 00 FF 02 02 10 00 CF 09 0C 07 E6 0B 08 FF 04 23 25 00 80 00 FF 02 02 10 00 85 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 87 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 71 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10

01 2D 09 0C 07 E6 0B 08 FF 05 0D 2A 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 05 0D 2A 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 06 33 08 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 07 1B 35 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 07 1C 18 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF

C0 02 81 00 00 00 01

C4 02 81

01

00 00 00 02

00

66

07 1C 22 00 80 00 FF 02 02 10 00 C9 09 0C 07 E6 0B 0F FF 10 19 0C 00 80 00 FF 02 02 10 00 66 09 0C 07 E6 0B 0F FF 10 19 0C 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 0F FF 10 19 15 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 0F FF 10 19 1C 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 0F FF 10 1A 3B 00 80 00 FF

# Event Read between two Dates (Block Transfer):

C0 01 C1 00 07 01 00 63 62 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E8 01 08 FF 13 33 00 FF FE D4 00 09 0C 07 E8 01 09 FF 13 33 00 FF FE D4 00 01 00

C4

02 C1

00

00 00 00 01

00 82

01 18

01 14 02 02 10 00 DA 09 0C 07 E6 0B 08 FF 04 22 38 00 80 00 FF 02 02 10 00 CF 09 0C 07 E6 0B 08 FF 04 23 25 00 80 00 FF 02 02 10 00 85 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 87 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 71 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10

01 2D 09 0C 07 E6 0B 08 FF 05 0D 2A 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 05 0D 2A 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 06 33 08 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 07 1B 35 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 07 1C 18 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF

C0 02 C1 00 00 00 01

C4 02 81

01

00 00 00 02

00

66

07 1C 22 00 80 00 FF 02 02 10 00 C9 09 0C 07 E6 0B 0F FF 10 19 0C 00 80 00 FF 02 02 10 00 66 09 0C 07 E6 0B 0F FF 10 19 0C 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 0F FF 10 19 15 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 0F FF

10 19 1C 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 0F FF 10 1A 3B 00 80 00 FF

# Event Read between two Dates (With Out Block Transfer):

C0 01 C1 00 07 01 00 63 62 00 FF 02 01 01 02 04 02 04 12 00 08 09 06 00 00 01 00 00 FF 0F 02 12 00 00 09 0C 07 E8 01 08 FF 13 33 00 FF FE D4 00 09 0C 07 E8 01 09 FF 13 33 00 FF FE D4 00 01 00

C4 01 81 00 01 0C 02 02 10 00 DA 09 0C 07 E6 0B 08 FF 04 22 38 00 80 00 FF 02 02 10 00 CF 09 0C 07 E6 0B 08 FF 04 23 25 00 80 00 FF 02 02 10 00 85 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 87 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 00 71 09 0C 07 E6 0B 08 FF 04 22 26 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 25 02 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 04 26 18 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 05 0D

2A 00 80 00 FF 02 02 10 01 2E 09 0C 07 E6 0B 08 FF 05 0D 2A 00 80 00 FF 02 02 10 01 2D 09 0C 07 E6 0B 08 FF 06 33 08 00 80 00 FF

# On Demand parameter read:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

## DMDT (Device Meta Data):

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 01 00 00 60 3C 06 FF 02 00

C4 01 81 00 02 02 09 03 0C 00 00 12 00 01

C0 01 81 00 01 00 00 60 3C 12 FF 02 00

C4 01 81 00 11 00

C0 01 81 00 01 00 00 5E 42 1E FF 02 00

C4 01 81 00 11 01

## WSIM (Wakeup Sim ID)

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 01 00 00 60 0C 80 FF 02 00

RCV: 00 01 00 01 00 30 00 11 C4 01 81 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 01 00 00 60 0C 81 FF 02 00

Rcv: 00 01 00 01 00 30 00 11 C4 01 81 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

Send: 00 01 00 30 00 01 00 0D C0 01 81 00 01 00 00 60 0C 82 FF 02 00

RCV: 00 01 00 01 00 30 00 11 C4 01 81 00 09 0B 30 33 33 33 34 37 35 39 36 39 39

## IPPO (Programmed IP and Port)

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 01 00 00 19 04 80 FF 02 00

C4 01 81 00 02 15 11 00 04 01 02 09 04 24 16 B9 F2 09 02 20 0D 09 04 24 16 B9 F2 09 02 20 0D 09 04 FF FF FF FF 09 02 FF FF 09 04 FF FF FF FF 09 02 FF FF 0A 01 30 09 04 FF FF FF FF 09 04 FF FF FF FF 0A 05 43 4D 4E 45 54 0A 01 30 0A 04 6E 75 6C 6C 0A 04 6E 75 6C 6C 11 0A 11 01 11 02 11 00

## MDSM (Programmed Meter Data Sampling)

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 07 01 00 63 01 00 FF 04 00

C4 01 81 00 06 00 00 02 58

C0 01 81 00 07 01 00 63 01 01 FF 04 00

C4 01 81 00 06 00 00 02 58

C0 01 81 00 07 01 00 63 02 00 FF 04 00

C4 01 81 00 06 00 00 02 58

C0 01 81 00 03 00 00 5E 5C 28 FF 02 00

C4 01 81 00 11 00

C0 01 81 00 01 00 00 5E 5C 2C FF 02 00

C4 01 81 00 09 0C 07 E7 09 09 03 12 39 00 00 80 00 00

## OPPO (Programmed Optical Port)

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 01 00 00 60 3C 10 FF 02 00

C4 01 81 00 02 02 09 0C 07 D0 01 01 06 00 00 00 00 80 00 FF 09 0C 07 E9 01 01 03 00 00 00 00 80 00 FF

C0 01 81 00 01 00 00 60 3C 10 FF 02 00

C4 01 81 00 02 02 09 0C 07 E2 0A 12 06 0D 0B 0B 00 80 00 FF 09 0C 08 02 0A 12 03 0D 0E 0F 00 80 00 FF

## MSIM (SIM -ID of Sim inside meter)

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 01 00 00 60 01 05 FF 02 00

C4 01 81 00 09 14 38 39 39 32 33 30 30 30 30 30 34 35 33 31 34 31 39 37 37 46

C0 01 81 00 01 00 00 60 01 06 FF 02 00

C4 01 81 00 09 0F 38 36 31 33 36 35 30 34 33 31 32 38 34 31 37

## AUXR (Aux Relay status)

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 46 00 00 60 03 0A FF 02 00

C4 01 81 00 03 01

C0 01 81 00 46 00 00 60 03 0A FF 03 00

C4 01 81 00 16 01

## Programmed Sanctioned Load:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 03 01 00 0F 23 00 00 02 00

C4 01 81 00 06 00 01 38 80

C0 01 81 00 03 01 00 0F 2C 00 00 02 00

C4 01 81 00 12 00 1E

C0 01 81 00 01 01 00 93 2D 00 FF 02 00

C4 01 81 00 11 01

C0 01 81 00 03 01 00 93 2C 00 FF 02 00

C4 01 81 00 12 00 0A

C0 01 81 00 03 01 00 93 2E 00 FF 02 00

C4 01 81 00 06 00 00 00 05

## Programmed Load shedding schedule:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

### With Out Block transfer response:

C0 01 81 00 0A 00 00 0C 00 00 FF 02 00

C4

01

81

00 01

05

02 0A 12 00 03 03 01 09 06 00 00 0A 00 00 FF 12 00 02 09 04 00 00 00 FF 12 00 00 04 07 00 04 09 00 01 09 05 FF FF 0A 1C FF 09 05 FF FF 0A 1E FF

02 0A 12 00 01 03 01 09 06 00 00 0A 00 00 FF 12 00 02 09 04 05 00 00 FF 12 00 00 04 07 00 04 09 00 61 09 05 FF FF 0A 1A FF 09 05 FF FF 0A 1A FF

02 0A 12 00 02 03 01 09 06 00 00 0A 00 00 FF 12 00 01 09 04 05 01 00 FF 12 00 00 04 07 00 04 09 00 81 09 05 FF FF 0A 1A FF 09 05 FF FF 0A 1A FF

02 0A 12 00 04 03 01 09 06 00 00 0A 00 00 FF 12 00 02 09 04 05 0A 00 FF 12 00 00 04 07 00 04 09 00 81 09 05 FF FF 0A 1C FF 09 05 FF FF 0A 1E FF

02 0A 12 00 05 03 01 09 06 00 00 0A 00 00 FF 12 00 01 09 04 05 0F 00 FF 12 00 00 04 07 00 04 09 00 01 09 05 FF FF 0A 1C FF 09 05 FF FF 0A 1E FF

C4 01 81 00 01

03

02 0A 12 00 03 03 01 09 06 00 00 0A 00 00 FF 12 00 02 09 04 03 00 00 FF 12 00 00 04 07 00 04 09 00 01 09 05 07 E7 09 09 FF 09 05 07 E7 09 0A FF

02 0A 12 00 02 03 01 09 06 00 00 0A 00 00 FF 12 00 02 09 04 09 00 00 FF 12 00 00 04 07 00 04 09 00 61 09 05 07 E7 09 09 FF 09 05 07 E7 09 0A FF

02 0A 12 00 01 03 01 09 06 00 00 0A 00 00 FF 12 00 01 09 04 17 00 00 FF 12 00 00 04 07 00 04 09 00 61 09 05 07 E7 09 09 FF 09 05 07 E7 09 0A FF

### With Block transfer response:

C0 01 81 00 0A 00 00 0C 00 00 FF 02 00

C4

02

81

00

00 00 00 01

00 82

01 18

01 0F

02 0A 12 00 01 03 01 09 06 00 00 0A 00 00 FF 12 00 02 09 04 00 00 00 FF 12 00 00 04 07 00 04 09 00 01 09 05 FF FF 0A 1C FF 09 05 FF FF 0A 1E FF

02 0A 12 00 02 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 01 00 00 FF 12 00 00 04 07 00 04 09 00 A1 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 03 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 02 00 00 FF 12 00 00 04 07 00 04 09 00 21 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 04 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 03 00 00 FF 12 00 00 04 07 00 04 09 00 21 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 05 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 04 00 00 FF 12 00 00 04 07 00 04 09 00 01 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 06 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 05 00 00 FF 12 00 00 04 07 00 04 09 00 61 09 05 FF FF

C0 02 81 00 00 00 01

C4

02

81

00

00 00 00 02

00 82

01 18

01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 07 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 06 00 00 FF 12 00 00 04 07 00 04 09 00 81 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 08 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 07 00 00 FF 12 00 00 04 07 00 04 09 00 81 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 09 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 08 00 00 FF 12 00 00 04 07 00 04 09 00 01 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 0A 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 09 00 00 FF 12 00 00 04 07 00 04 09 00 81 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 0B 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 0A 00 00 FF 12 00 00 04 07 00 04 09 00 61 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 0C 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 0B 00 00 FF 12 00 00 04 07 00

C0 02 81 00 00 00 02

C4 02 81 01 00 00 00 03

00 81

A2

04 09 00 81 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 0D 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 0C 00 00 FF 12 00 00 04 07 00 04 09 00 01 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 0E 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 0D 00 00 FF 12 00 00 04 07 00 04 09 00 21 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

02 0A 12 00 0F 03 00 09 06 00 00 0A 00 00 FF 12 00 01 09 04 0E 00 00 FF 12 00 00 04 07 00 04 09 00 21 09 05 FF FF 01 01 FF 09 05 FF FF 01 01 FF

## MDI

C0 01 81 00 16 00 00 0F 00 00 FF 04 00

C4 01 81 00 01 01 02 02 09 04 0B 15 19 00 09 05 FF FF FF 05 FF

## Programmed Time of Use:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame

C0 01 81 00 14 00 00 0D 00 00 FF 0A 00

C4 01 81 00 09 0C 07 DF 01 01 00 00 00 00 00 00 00 00

Active Profile:

C0 01 C1 00 14 00 00 0D 00 00 FF 02 00

C4 01 C1 00 09 06 30 30 30 30 30 30

C0 01 C1 00 14 00 00 0D 00 00 FF 03 00

C4 01 C1

00 01

04

02 03 09 01 01 09 0C FF FF 0C 01 FF FF FF FF FF 80 00 00 09 01 01

02 03 09 01 02 09 0C FF FF 03 01 FF FF FF FF FF 80 00 00 09 01 02

02 03 09 01 03 09 0C FF FF 06 01 FF FF FF FF FF 80 00 00 09 01 03

02 03 09 01 04 09 0C FF FF 09 01 FF FF FF FF FF 80 00 00 09 01 04

C0 01 C1 00 14 00 00 0D 00 00 FF 04 00

C4 01 C1

00 01

04

02 08 09 01 01 11 01 11 01 11 01 11 01 11 01 11 01 11 01

02 08 09 01 02 11 02 11 02 11 02 11 02 11 02 11 02 11 02

02 08 09 01 03 11 03 11 03 11 03 11 03 11 03 11 03 11 03

02 08 09 01 04 11 04 11 04 11 04 11 04 11 04 11 04 11 04

C0 01 C1 00 14 00 00 0D 00 00 FF 05 00

C4 01 C1

00 01

04

02 02 11 01 01 02 02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 02 01 02 02 03 09 04 12 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 16 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 03 01 02 02 03 09 04 13 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 17 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 04 01 02 02 03 09 04 12 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 16 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

Read DAY profile (Block transfer):

C0 01 C1 00 14 00 00 0D 00 00 FF 05 00

C4 02 C1

00

00 00 00 01

00 82

01 18

01 07

02 02 11 01 01 03 02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 03

02 02 11 02 01 02 02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 03 01 02 02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 04 01 02 02 03 09 04 12 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 16 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 05 01 02 02 03 09 04 12 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 14 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 06 01 02 02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00

C0 02 C1 00 00 00 01

C4 02 C1

01

00 00 00 02

00 44

64 FF 12 00 02

02 02 11 07 01 03 02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 03

Read Holiday Profile without Block Transfer:

C0 01 81 00 0B 00 00 0B 00 00 FF 02 00

C4 01 81 00 01 09 02 03 12 00 01 09 05 FF FF 01 01 FF 11 05 02 03 12 00 02 09 05 FF FF 02 01 FF 11 05 02 03 12 00 03 09 05 FF FF 03 01 FF 11 05 02 03 12 00 04 09 05 FF FF 04 01 FF 11 05 02 03 12 00 05 09 05 FF FF 05 01 FF 11 05 02 03 12 00 06 09 05 FF FF 06 01 FF 11 05 02 03 12 00 07 09 05 FF FF 07 01 FF 11 05 02 03 12 00 08 09 05 FF FF 08 01 FF 11 05 02 03 12 00 09 09 05 FF FF 09 01 FF 11 05

Read Holiday Profile Block Transfer:

C0 01 81 00 0B 00 00 0B 00 00 FF 02 00

C4 02 81 00 00 00 00 01 00 82 01 18 01 1D 02 03 12 00 01 09 05 FF FF 01 01 FF 11 05 02 03 12 00 02 09 05 FF FF 01 03 FF 11 05 02 03 12 00 03 09 05 FF FF 01 05 FF 11 05 02 03 12 00 04 09 05 FF FF 02 01 FF 11 05 02 03 12 00 05 09 05 FF FF 02 03 FF 11 05 02 03 12 00 06 09 05 FF FF 02 05 FF 11 05 02 03 12 00 07 09 05 FF FF 03 01 FF 11 05 02 03 12 00 08 09 05 FF FF 03 03 FF 11 05 02 03 12 00 09 09 05 FF FF 03 05 FF 11 05 02 03 12 00 0A 09 05 FF FF 04 01 FF 11 05 02 03 12 00 0B 09 05 FF FF 04 03 FF 11 05 02 03 12 00 0C 09 05 FF FF 04 05 FF 11 05 02 03 12 00 0D 09 05 FF FF 05 01 FF 11 05 02 03 12 00 0E 09 05 FF FF 05 03 FF 11 05 02 03 12 00 0F 09 05 FF FF 05 05 FF 11 05 02 03 12 00 10 09 05 FF FF 06 01 FF 11 05 02 03 12 00 11 09 05 FF FF 06 03 FF 11 05 02 03 12 00 12 09 05 FF FF 07 01 FF 11 05 02 03 12 00 13 09 05 FF FF 07 03 FF 11 05 02 03 12 00 14 09 05 FF FF 08 01 FF

C0 02 81 00 00 00 01

C4 02 81 01 00 00 00 02 00 81 80 11 05 02 03 12 00 15 09 05 FF FF 08 03 FF 11 05 02 03 12 00 16 09 05 FF FF 09 01 FF 11 05 02 03 12 00 17 09 05 FF FF 09 03 FF 11 05 02 03 12 00 18 09 05 FF FF 0A 01 FF 11 05 02 03 12 00 19 09 05 FF FF 0A 03 FF 11 05 02 03 12 00 1A 09 05 FF FF 0B 01 FF 11 05 02 03 12 00 1B 09 05 FF FF 0B 03 FF 11 05 02 03 12 00 1C 09 05 FF FF 0C 01 FF 11 05 02 03 12 00 1D 09 05 FF FF 0C 03 FF 11 05

## OVFC:

C0 01 81 00 03 01 00 0C 23 00 01 02 00

C4 01 81 00 06 00 00 6B D0

## UVFC

C0 01 81 00 03 01 00 0C 1F 00 01 02 00

C4 01 81 00 06 00 00 13 88

## 14-OCFC

C0 01 81 00 03 01 00 0B 23 00 00 02 00

C4 01 81 00 06 00 00 2E E0

## 15-OLFC

C0 01 81 00 03 01 00 0F 23 00 04 02 00

C4 01 81 00 06 00 00 20 58

## 16-VUFC

C0 01 81 00 03 01 00 92 23 01 01 02 00

C4 01 81 00 06 00 00 00 0F

## 17-PFFC

C0 01 81 00 03 01 00 0C 27 00 01 02 00

C4 01 81 00 06 00 00 00 00

## 18-CUFC

C0 01 81 00 03 01 00 92 23 02 01 02 00

C4 01 81 00 06 00 00 00 0F

## 19-HAPF

C0 01 81 00 03 01 00 09 23 00 01 02 00

C4 01 81 00 06 00 00 1A F4

# Device Creation:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C1 01 81 00 01 00 00 60 3C 06 FF 02 00 02 02 09 03 0C 00 00 12 00 01

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 12 FF 02 00 11 00

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 0B FF 02 00 03 00

C5 01 81 00

C1 01 81 00 16 00 00 0F 00 00 FF 04 00 01 02 02 04 09 04 0F 12 00 FF 09 05 FF FF FF 12 FF

C5 01 81 00

C1 01 81 00 01 00 00 5E 42 1E FF 02 00 11 02

C5 01 81 00

Event management commands:

C1 01 81 00 03 01 00 92 23 00 FF 02 00 06 00 00 00 02

C5 01 81 00

C1 01 81 00 03 01 00 92 2C 00 FF 02 00 12 00 03

C5 01 81 00

# Update Meta Data:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C1 01 81 00 01 00 00 60 3C 06 FF 02 00 02 02 09 03 0C 00 00 12 00 01

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 12 FF 02 00 11 00

C5 01 81 00

C1 01 81 00 01 00 00 5E 42 1E FF 02 00 11 02

C5 01 81 00

# Update MDI Reset Date:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C1 01 81 00 01 00 00 60 3C 0B FF 02 00 03 00

C5 01 81 00

C1 01 81 00 16 00 00 0F 00 00 FF 04 00 01 02 02 04 09 04 0F 12 00 FF 09 05 FF FF FF 12 FF

C5 01 81 00

# Update IP Port:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C1 02 81 00 01 00 00 19 04 80 FF 02 00 00 00 00 00 01 47 02 15 11 00 04 01 02 09 04 77 A0 6B 6B 09 02 7D DC 09 04 77 A0 6B 6B 09 02 7D DC 09 04 FF FF FF FF 09 02 FF FF 09 04 FF FF FF FF 09 02 FF FF 0A 01 30 09 04 FF FF FF FF 09 04 FF FF FF FF 0A 05 43 4D 4E 45 54 0A 01

C5 02 81 00 00 00 01

C1 03 81 FF 00 00 00 02 15 30 0A 04 6E 75 6C 6C 0A 04 6E 75 6C 6C 11 0A 11 00 11 01 11 00

C5 03 81 00 00 00 00 02

# Meter Data Sampling:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

C1 01 81 00 03 00 00 5E 5C 29 FF 02 00 06 00 00 00 65

C5 01 81 00

C1 01 81 00 03 00 00 5E 5C 2A FF 02 00 06 00 00 00 66

C5 01 81 00

C1 01 81 00 03 00 00 5E 5C 2B FF 02 00 06 00 00 00 67

C5 01 81 00

C1 01 81 00 03 00 00 5E 5C 28 FF 02 00 11 02

C5 01 81 00

C1 01 81 00 01 00 00 5E 5C 2C FF 02 00 09 0C 07 E7 05 0A 03 0A 0B 0C 00 80 00 00

C5 01 81 00

# Aux Relay operation:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

Disconnect Command:

C3 01 81 00 46 00 00 60 03 0A FF 01 01 0F 00

C7 01 81 00 00

Reconnect Command:

C3 01 81 00 46 00 00 60 03 0A FF 02 01 0F 00

C7 01 81 00 00

# Sanctioned Load Control:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

The meter support T1, T2, T3, T4 tariff load side management. But in UDIL there is one parameter load_limit so pass this value to all registers.

C1 01 81 00 01 00 00 60 3C 0A FF 02 00 03 01

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 0A FF 02 00 03 01

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 00 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 00 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 01 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 01 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 02 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 02 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 03 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 03 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 01 01 00 93 2D 00 FF 02 00 11 03

C5 01 81 00

C1 01 81 00 03 01 00 93 2C 00 FF 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 03 01 00 93 2E 00 FF 02 00 06 00 00 00 1E

C5 01 81 00

# Load_shedding_Schedule:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

Developer Need to program Load shedding schedule one by one as below and can save index function upto 30. Start date and end date should not be same e.g. if one day load shedding is required then program start date would be 03/04 to 04/04

Send Once

{

C3 01 C1 00 09 00 00 0A 00 00 FF 01 01 12 00 0B

C7 01 C1 00 00

}

C3 01 81 00 0A 00 00 0C 00 00 FF 02 01 02 0A 12 00 04 03 01 09 06 00 00 0A 00 00 FF 12 00 01 09 04 10 0F 14 FF 12 00 00 04 07 00 04 00 09 05 07 E7 0B 01 FF 09 05 07 E7 0B 1E FF

C7 01 81 00 00

# Paramterization_Cancellation:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

Upon every cancellation command HES will generate cancellation event in DB with respect to event.

## SANC:

C1 01 81 00 01 00 00 60 3C 0A FF 02 00 03 01

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 00 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 00 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 01 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 01 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 02 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 02 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 03 02 00 06 00 01 0D 88

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 03 02 00 12 00 1E

C5 01 81 00

C1 01 81 00 01 01 00 93 2D 00 FF 02 00 11 03

C5 01 81 00

C1 01 81 00 03 01 00 93 2C 00 FF 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 03 01 00 93 2E 00 FF 02 00 06 00 00 00 00

C5 01 81 00

## LSCH:

C3 01 C1 00 09 00 00 0A 00 00 FF 01 01 12 00 0B

C7 01 C1 00 00

## TIOU:

C3 01 C1 00 09 00 00 0A 00 00 FF 01 01 12 00 0C

C7 01 C1 00 00

## OVFC:

C1 01 81 00 03 01 00 0C 23 00 01 02 00 06 00 00 6B D0

C5 01 81 00

## UVFC:

C1 01 81 00 03 01 00 0C 1F 00 01 02 00 06 00 00 13 88

C5 01 81 00

## OCFC:

C1 01 81 00 03 01 00 0B 23 00 00 02 00 06 00 00 2E E0

C5 01 81 00

## OLFC:

C1 01 81 00 03 01 00 0F 23 00 04 02 00 06 00 00 20 58

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 05 02 00 06 00 00 20 58

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 06 02 00 06 00 00 20 58

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 07 02 00 06 00 00 20 58

C5 01 81 00

## VUFC:

C1 01 81 00 03 01 00 92 23 01 01 02 00 06 00 00 00 0F

C5 01 81 00

## PFFC:

C1 01 81 00 03 01 00 0C 27 00 01 02 00 06 00 00 00 00

C5 01 81 00

## CUFC:

C1 01 81 00 03 01 00 92 23 02 01 02 00 06 00 00 00 0F

C5 01 81 00

## HAPF:

C1 01 81 00 03 01 00 09 23 00 01 02 00 06 00 00 1A F4

C5 01 81 00

# Update Time of Use:

C1 01 81 00 14 00 00 0D 00 00 FF 0A 00 09 0C 07 E6 0B 08 02 0C 37 00 00 80 00 00

C5 01 81 00

## Holiday Profile

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

If the Holiday numbers are 4 or less it would be sent as without block transfer otherwise would be sent as block transfer.

### Holiday Profile Without block transfer:

C1 01 81 00 0B 00 00 0B 00 00 FF 02 00 01 04 02 03 12 00 05 09 05 FF FF 01 01 FF 11 05 02 03 12 00 05 09 05 FF FF 05 02 FF 11 05 02 03 12 00 05 09 05 FF FF 08 01 FF 11 05 02 03 12 00 05 09 05 FF FF 08 11 FF 11 05

C1 01 81 00 0B 00 00 0B 00 00 FF 02 00 01 03 02 03 12 00 05 09 05 FF FF 17 03 FF 11 04 02 03 12 00 05 09 05 FF FF 0E 08 FF 11 04 02 03 12 00 05 09 05 FF FF 05 02 FF 11 04

C5 01 81 00

### Holiday Profile Block Transfer:

C1 02 81 00 0B 00 00 0B 00 00 FF 02 00 00 00 00 00 01 47 01 0A 02 03 12 00 05 09 05 FF FF 01 01 FF 11 05 02 03 12 00 05 09 05 FF FF 02 01 FF 11 05 02 03 12 00 05 09 05 FF FF 03 01 FF 11 05 02 03 12 00 05 09 05 FF FF 04 01 FF 11 05 02 03 12 00 05 09 05 FF FF 05 02 FF 11

C5 02 81 00 00 00 01

C1 03 81 FF 00 00 00 02 47 05 02 03 12 00 05 09 05 FF FF 06 01 FF 11 05 02 03 12 00 05 09 05 FF FF 07 01 FF 11 05 02 03 12 00 05 09 05 FF FF 08 01 FF 11 05 02 03 12 00 05 09 05 FF FF 09 01 FF 11 05 02 03 12 00 05 09 05 FF FF 0A 01 FF 11 05

C5 03 81 00 00 00 00 02

## Season Profile:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

If the Season numbers are 3 or less it would be sent as without block transfer otherwise would be sent as block transfer.

### Season Profile Without block transfer:

C1 01 C1 00 14 00 00 0D 00 00 FF 07  00 01 03 02 03 09 01 01 09 0C FF FF 0C 01 FF FF FF FF FF 80 00 00  09 01 01 02 03 09 01 02 09 0C FF FF 03 01 FF FF FF FF FF 80 00 00 09 01 02 02 03 09 01 03 09 0C FF FF 06 01 FF FF FF FF FF FE D4 00 09 01 03

C5 01 C1 00

### Season Profile Block Transfer:

C1 02 81 00 14 00 00 0D 00 00 FF 07 00 00 00 00 00 01 47 01 04 02 03 09 01 01 09 0C FF FF 0C 01 FF FF FF FF FF 80 00 00 09 01 01 02 03 09 01 02 09 0C FF FF 03 01 FF FF FF FF FF 80 00 00 09 01 02 02 03 09 01 03 09 0C FF FF 06 01 FF FF FF FF FF 80 00 00 09 01 03 02 03 09

C5 02 81 00 00 00 01

C1 03 81 FF 00 00 00 02 13 01 04 09 0C FF FF 09 01 FF FF FF FF FF 80 00 00 09 01 04

C5 03 81 00 00 00 00 02

## Week Profile:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

the Week numbers could not be more than 4.

### Week Profile:

C1 01 C1 00 14 00 00 0D 00 00 FF 04 00 01 04 02 08 09 01 01 11 01 11 01 11 01 11 01 11 01 11 01 11 01 02 08 09 01 02 11 02 11 02 11 02 11 02 11 02 11 02 11 02 02 08 09 01 03 11 03 11 03 11 03 11 03 11 03 11 03 11 03 02 08 09 01 04 11 04 11 04 11 04 11 04 11 04 11 04 11 04

C5 01 C1 00

C1 02 81 00 14 00 00 0D 00 00 FF 08 00 00 00 00 00 01 47 01 04 02 08 09 01 01 11 01 11 01 11 01 11 01 11 01 11 01 11 01 02 08 09 01 02 11 02 11 02 11 02 11 02 11 02 11 02 11 02 02 08 09 01 03 11 03 11 03 11 03 11 03 11 03 11 03 11 03 02 08 09 01 04 11 04 11 04 11 04 11

## Day Profile:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

If the Days numbers are 2 or less it would be sent as without block transfer otherwise would be sent as block transfer.

### Day Profile Without Block Transfer:

C1 01 C1 00 14 00 00 0D 00 00 FF 09 00 01 02 02 02 11 01 01 02 02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02 02 02 11 02 01 02 02 03 09 04 12 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 16 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

C5 01 C1 00

### Day Profile Block Transfer:

C1 02 81 00 14

00 00 0D 00 00 FF 09 00 00 00 00 00 01

47 01 04

02 02

11 01

01 02

02 03 09 04 11 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01 02 03 09 04 15 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02 02 02 11 02 01 02 02 03 09 04 12 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01

C5 02 81 00 00 00 01

C1 03 81 00 00 00 00 02

47

02 03 09 04 16 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 03

01 02

02 03 09 04 13 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01

02 03 09 04 17 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 04 01 02 02 03

C5 02 81 00 00 00 02

C1 03 81 FF 00 00 00 03

2A

09 04 12 00 00 00 09 06 00 00 0A 00 64 FF 12 00 01

02 03

09 04 16 00 00 00 09 06 00 00 0A 00 64 FF 12 00 02

02 02 11 05 01 00

C5 03 81 00 00 00 00 03

# APMS Tripping Events:

Blue is sent frame please add [ 00 01 00 30 00 01 00 length_of_frame] before sending each frame to meter.

Green is received frame [00 01 00 01 00 30 00 length_of_frame] would be there at start of frame.

## OVFC:

C1 01 81 00 03 01 00 0C 23 00 FF 02 00 06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0C 2C 00 FF 02 00 12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0C 23 00 01 02 00 06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0C 2C 00 01 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 15 FF 02 00 03 01

C5 01 81 00

## UVFC:

C1 01 81 00 03 01 00 0C 1F 00 FF 02 00 06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0C 2B 00 FF 02 00 12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0C 1F 00 01 02 00 06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0C 2B 00 01 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 16 FF 02 00 03 01

C5 01 81 00

## OCFC:

C1 01 81 00 03 01 00 0B 23 00 01 02 00 06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0B 2C 00 01 02 00 12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0B 23 00 00 02 00 06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0B 2C 00 02 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 17 FF 02 00 03 01

C5 01 81 00

## OLFC:

C1 01 81 00 03 01 00 0F 23 00 00 02 00 06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 2C 00 00 02 00 12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 23 00 04 02 00 06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 04 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 01 02 00  06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 2C 00 01 02 0012 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 23 00 05 02 00  06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 05 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 02 02 00  06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 2C 00 02 02 0012 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 23 00 06 02 00  06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 06 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 03 01 00 0F 23 00 03 02 00  06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 2C 00 03 02 0012 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0F 23 00 07 02 00  06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0F 2C 00 07 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 18 FF 02 00 03 01

C5 01 81 00

## VUFC:

C1 01 81 00 03 01 00 92 23 01 FF 02 00 06 00 00 00 0F

C5 01 81 00 36

C1 01 81 00 03 01 00 92 2C 01 FF 02 00 12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 92 23 01 01 02 00 06 00 00 00 0F

C5 01 81 00

C1 01 81 00 03 01 00 92 2C 01 01 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 1A FF 02 00 03 01

C5 01 81 00

## CUFC:

C1 01 81 00 03 01 00 92 23 02 FF 02 00 06 00 00 00 0F

C5 01 81 00 36

C1 01 81 00 03 01 00 92 2C 02 FF 02 00 12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 92 23 02 01 02 00 06 00 00 00 0F

C5 01 81 00

C1 01 81 00 03 01 00 92 2C 02 01 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 1B FF 02 00 03 01

C5 01 81 00

## PFFC:

C1 01 81 00 03 01 00 0C 27 00 FF 02 00 06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 0C 2D 00 FF 02 00  12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 0C 27 00 01 02 00 06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 0C 2D 00 01 02 00 12 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 19 FF 02 00 03 01

C5 01 81 00

## HAPF:

C1 01 81 00 03 01 00 09 23 00 FF 02 00 06 00 00 61 A8

C5 01 81 00 36

C1 01 81 00 03 01 00 09 2C 00 00 02 00 12 00 1E

C5 01 81 00 36

C1 01 81 00 03 01 00 09 23 00 01 02 00 06 00 00 6B D0

C5 01 81 00

C1 01 81 00 03 01 00 09 2C 00 01 02 0012 00 3C

C5 01 81 00

C1 01 81 00 01 00 00 60 3C 1C FF 02 00 03 01

C5 01 81 00

# Appendix I – DLMS Abbreviations

| Smart Meter Status | Description |
| --- | --- |
| DLMS Client | MDC |
| DLMS Server | Meter |
| MSN | Meter Serial Number |
| Mode 2 | Always On Mode |
| LLS | Low level security |
| RLRQ | Release Request |
| RLRE | Release Response |
| xDLMS |  |
| LN | Logical Name |
| AM5 | Authentication mechanism 5 |

Data::= CHOICE

{

null-data                          [0]   IMPLICIT   NULL,

array                              [1]   IMPLICIT   SEQUENCE OF Data,

structure                          [2]   IMPLICIT   SEQUENCE OF Data,

boolean                            [3]   IMPLICIT   BOOLEAN,

bit-string                         [4]   IMPLICIT   BIT STRING,

double-long                        [5]   IMPLICIT   Integer32,

double-long-unsigned               [6]   IMPLICIT   Unsigned32,

octet-string                       [9]   IMPLICIT   OCTET STRING,

visible-string                     [10]  IMPLICIT   VisibleString,

utf8-string                        [12]  IMPLICIT   UTF8String,

bcd                                [13]  IMPLICIT   Integer8,

integer                            [15]  IMPLICIT   Integer8,

long                               [16]  IMPLICIT   Integer16,

unsigned                           [17]  IMPLICIT   Unsigned8,

long-unsigned                      [18]  IMPLICIT   Unsigned16,

compact-array                      [19]  IMPLICIT   SEQUENCE

{

contents-description                [0]              TypeDescription,

array-contents                      [1]   IMPLICIT   OCTET STRING

},

long64                             [20]  IMPLICIT   Integer64,

long64-unsigned                    [21]  IMPLICIT   Unsigned64,

enum                               [22]  IMPLICIT   Unsigned8,

float32                            [23]  IMPLICIT   OCTET STRING (SIZE(4)),

float64                            [24]  IMPLICIT   OCTET STRING (SIZE(8)),

date-time                          [25]  IMPLICIT   OCTET STRING (SIZE(12)),

date                               [26]  IMPLICIT   OCTET STRING (SIZE(5)),

time                               [27]  IMPLICIT   OCTET STRING (SIZE(4)),

dont-care                          [255] IMPLICIT   NULL

}

-- The following TypeDescription relates to the compact-array data Type

Type Description ::= CHOICE

{

null-data                          [0]   IMPLICIT  NULL,

array                              [1]   IMPLICIT  SEQUENCE

{

number-of-elements        Unsigned16,

type-description          TypeDescription

},

structure                          [2]   IMPLICIT  SEQUENCE OF TypeDescription,

boolean                            [3]   IMPLICIT  NULL,

bit-string                         [4]   IMPLICIT  NULL,

double-long                        [5]   IMPLICIT  NULL,

double-long-unsigned               [6]   IMPLICIT  NULL,

octet-string                       [9]   IMPLICIT  NULL,

visible-string                     [10]  IMPLICIT  NULL,

utf8-string                        [12]  IMPLICIT  NULL,

bcd                                [13]  IMPLICIT  NULL,

integer                            [15]  IMPLICIT  NULL,

long                               [16]  IMPLICIT  NULL,

unsigned                           [17]  IMPLICIT  NULL,

long-unsigned                      [18]  IMPLICIT  NULL,

long64                             [20]  IMPLICIT  NULL,

long64-unsigned                    [21]  IMPLICIT  NULL,

enum                               [22]  IMPLICIT  NULL,

float32                            [23]  IMPLICIT  NULL,

float64                            [24]  IMPLICIT  NULL,

date-time                          [25]  IMPLICIT  NULL,

date                               [26]  IMPLICIT  NULL,

time                               [27]  IMPLICIT  NULL,

dont-care                          [255] IMPLICIT  NULL

}

Data-Access-Result ::= ENUMERATED

{

success                            (0),

hardware-fault                     (1),

temporary-failure                  (2),

read-write-denied                  (3),

object-undefined                   (4),

object-class-inconsistent          (9),

object-unavailable                 (11),

type-unmatched                     (12),

scope-of-access-violated           (13),

data-block-unavailable             (14),

long-get-aborted                   (15),

no-long-get-in-progress            (16),

long-set-aborted                   (17),

no-long-set-in-progress            (18),

data-block-number-invalid          (19),

other-reason                       (250)

}

## Figures

### AARQ request format:

| Wrapper | AARQ | X DLMS | Conformance Block |
| --- | --- | --- | --- |

Figure 1

### Application Layer Services summary for LN referencing:

Figure 2

### State Machine for the client Side Control Function:

Figure 3

### State Machine for Server side Control Function:

Figure 4

To be Remember:

- Yellow Tag field need to be confirm form Shahab Sb.

- Develop cross reference.

Question while writing the document.

- What is the Encoding how it relate to particular field? Is it programmable/fix? See example of Encoding of xDLMS and AARQ.

Ans: Fix the encoding until European meter comes in Pakistan.

- What should be the datatype?

Octet string for bytes and others.

- How to read ASN.1 file and its relation with database?

No satisfactory answer. Given some clues for further study in skype

- How this file is used in Software development?

No satisfactory Answer.

- How Ciphering be implemented in this document?

Ans: This will be handled in MDC development. Not in this document.

- Do we need to follow MTI sequence of data read or make independent module?

Ans: Make independent Module.

- What is Octet 4 in reference 3?

- HLS with Gmac ciphering AARQ?

Ans: 000100010001007B

60

79

A1

09 06

07

60857405080103

A6

0A

04

08

4354543030303030

8A

02

07

80

8B

07

60857405080205

AC

2E

80

2C

Ctos:CFF26E9860F89D7A36E7C455B9395CF5A75FDDAD427AF3D2D5F5492B1C850FE84B1784EDA0C33B72CECBDC55

BE

23

04

21

XDLMs: 211F303A9821932886F5AF45E00BEDCF6AF1348937F1F74878AF60216EF561D1BA

- HLS with Gmac AARE?

Ans: 0001000100010088

61

81

85

A1

09

06

07

60857405080103

A2

03

02

01

00

A3

05

A1

03

02

01

0E

A4

0A

04

08

4143430371C9F3B9

88

02

07

80

89

07

60857405080205

AA

2E

80

2C

Stoc: 3043323339393941433934423745454141303339313539373833423641374632324445463146364144314134

BE

23

04

21

281F300000000088D57492AE66431226C6CB1490B18CA89F9D6145DD3FF48C0CCB

- What is the XML encoding rules to ASN.1 and XML schema definition language XSD? Does the following table have some connection to this knowledge/rules?

Ans: As per Shahab Sb these are the services states written see book 224 page.

- What type of communication method you want to use for data communication i.e, block type, without block or list etc. Especially Access?

Ans: Read Meter block wise and Shahab said verify the channel writing command? Read Action for configuration?

- What is the selective service access for message communication?

Ans:  it is partially calling of main service.

- Design a Relational Database management System based on this document. The database system may be MySQL etc.

Ans: Shahab sb will provide RDMS.

- What is PH-data.Request/.indication (data transfer service)? What is the role of this service in obtaining AA with unknown server? Reference

Ans: Non MVP minimum viable product.

- Data of inst, loadprofile, and billing is stored in channel. How to read a channel.

Ans: read the meter channel and get the frame.

- What is the difference between read and Access?

Ans: Read is the value of attribute while the Access is the accessing of objects only.